GE220-UK-2RS-B

Spherical plain bearing

Radial spherical plain bearing, maintenance-free, sliding layer: ELGOGLIDE, inner ring curved surface with hard chromium coating; DIN ISO 12240-1; dimension series E, sealed

Add to wish list

Contact us

Product informationCalculationsCADMounting

Main Dimensions & Performance DataDownload data sheet

D

320 mm

Outside diameter bearing

d

220 mm

Bore diameter bearing

B

135 mm

Width Inner ring

C

100 mm

Width Outer ring

Cr

6,600,000 N

Basic dynamic load rating, radial

C0r

11,000,000 N

Basic static load rating, radial

32 kg

Weight

Dimensions

dK

275 mm

Ball diameter

α

8 °

Tilt angle

Gr

0 - 0,11 mm

Radial Clearance

dUT

-0.03 mm

Bore diameter bearing, lower tolerance

DUT

-0.04 mm

Outside diameter, lower tolerance

BUT

-0.3 mm

Width inner ring, lower tolerance

COT

-0.8 mm

Width outer ring, lower tolerance

Mounting dimensions

r1smin

1.1 mm

Edge Spacing

r2smin

1.1 mm

Edge Spacing

da max

239.5 mm

Connection measure Inner ring

Da min

267 mm

Housing Connection Diameter

Temperature range

Tmin

-30 °C

Operating temperature min.

Tmax

130 °C

Operating temperature max.